CENG 450 Computer Systems & Architecture Quiz #2 (Feb. 10, 2003)

Name:_______________________________	Total Marks: 60.

- Consider the  MIPS code listed below.

- List all data and control dependencies. For each data dependency, tell whether it’s a true or name dependency. (8 points)

- Assuming that R1 and R2 point to two arrays (e.g., X[i] and Y[i]) and that F2 stores a scalar (e.g., A),  explain what the code does. (12 points)

LOOP: LD  F0, 0(R1)

MULTD F0, F0, F2

LD F4, 0(R2)

ADD F0,F0,F4

SD 0(R2),F0

SUBI R1,R1,8

SUBI R2,R2,8

BNEZ R1, LOOP

Name:_______________________________

2. Consider the following code:

ADD                    R1, R2, R3

SUB                     R4, R6, R2

LD                        R8, 10(R16)

ADD                     R9, R8, R1

## SD                        10(R16), R9

MULT                  R6, R1,R4

ADD                     R4, R8, R1

- Assume that all instructions but MULT take 2 cycles and MULT takes 10 cycles. Also, Assume that we have infinite number of resources and therefore structural hazards never happen. Answer the followings

- How long does it take to execute the code if pipelining was not used (5 points)?

- Determine when every instruction issues, reads operands, completes execution and writes back if the Scoreboard techniques is used. Use a table similar to that used in the class(15 points).

- Determine when every instruction issues, completes execution and writes back if the Tomasulo techniques is used. Use a table similar to that used in the class(15 points).

- Compare your answers to parts b and c. Listing the differences between Tomasulo and Scoreboard explain why exactly results are different. (5 points).

Name:_______________________________

Name:_______________________________