University Of Victoria

Final Examination – December 2006

ELEC 200 – Engineering Graphics

Section F01

Answer Sheet

- (20%) Let pv = [xv, yv, zv, 1]T be the view point and let po = [xo, yo, zo, 1]T be the object point.  Derive the perspective projection which views po from pv and write it as a product of elementary translation(s), rotation(s), perspective(s) and projection(s).  Draw one or more diagrams which clearly show(s) every relevant point, line, distance and angle.

| To translates po to the origin. | Rxy() rotates p into the yz plane and Ryz() aligns the result with the z axis. p |
| --- | --- |

We have  To=100−xo010−yo001−zo0001,  which takes pv into p, defined by

p=xyz1=Topv=100−xo010−yo001−zo0001xvyvzv1=xv−xoyv−yozv−zo1

Now define  h=x2+y2  and  r=x2+y2+z2, so that

sinθ=xh, cosθ=yh, Rxy(θ)=yh−xh00xhyh0000100001, and

sinφ=hr, cosφ=zr, Ryz(φ)=10000zr−hr00hrzr00001.

Applying these transforms takes the viewpoint into the point [0, 0, r, 1]T and we can immediately write the perspective transformation, Vz, looking at the origin from this point and the orthogonal projection, Pz, onto the xy plane as

Vz=10000100001000−hr1,   and   Pz=1000010000000001

The perspective projection sought is just the product of all the above transforms, in order of appearance:

P=PzVzRyz(φ)Rxy(θ)To

- (15%) Wite a MatLab script that animates the following rolling motion over a total of N frames:

Using 2D homogeneous coordinates, you may assume that you have a 3×N matrix, Q, containing the vertices of the object in columns, such that the call

h = patch( …

'vertices', [Q(1:2,:); zeros(1,size(Q,2)]', 'faces', [1:size(Q,2)] …

);

will create a patch object with a single face having the shape of the object which is to be transformed, placed so that the center of the circular part of the object is at the origin.  Take the radius of the circular part of the object to be r.  Use drawnow() instead of getframe().

% assume we have Q, h and N as defined in the question

R = [ % rotation for one frame

cos(pi/N) -sin(pi/N)  0

sin(pi/N)  cos(pi/N)  0

0          0          1

];

T = [ % translation for one frame

1  0 -r*pi/N

0  1  0

0  0  1

];

c = [r*pi/2 r 1]'; % initial center of the circular part

Tc = [ % translation from the origin to the initial center

1  0  c(1)

0  1  c(2)

0  0  1

];

Q = Tc * Q; % place the object in its initial position

set(h, 'vertices', [Q(1:2,:); zeros(1,size(Q,2))]');

for i = 1:N

To = [ % translate from current center to origin

1  0 -c(1)

0  1 -c(2)

0  0  1

];

Ti = [ % translate back to current center

1  0  c(1)

0  1  c(2)

0  0  1

];

Q = T * Ti * R * To * Q; % rotate and translate object

c = T * c; % translate the center

set(h, 'vertices', [Q(1:2,:); zeros(1,size(Q,2))]');

drawnow;

end

- Consider translations and rotations in three dimensions, expressed as 4x4 homogeneous matrices.

a)	(10%) Prove that translation and rotation do not commute in the general case.

Let T=100x010y001z0001 be an arbitrary translation and

let R=a11a12a130a21a22a230a31a32a3300001 be an arbitrary rotation.  Then, by direct

calculation, we have

RT=a11a12a130a21a22a230a31a32a3300001100x010y001z0001=a11a12a13a11x+a12y+a13za21a22a23a21x+a22y+a23za31a32a33a31x+a32y+a33z0001

but

TR=100x010y001z0001a11a12a130a21a22a230a31a32a3300001=a11a12a13xa21a22a23ya31a32a33z0001

since these are clearly not equal in the general case, we have the desired result that T and R do not commute in general.

b)	(5%) Prove that they do commute in the specific case when the translation is perpendicular to the plane of the rotation.  Hint: Use your freedom to choose the coordinate system to simplify the algebra.

Choose the coordinate system so that the rotation is in, say, the xy plane, in which case the translation must be along the z axis.

We have T=10000100001z0001 and R=a11a1200a21a220000100001, whereby

RT=a11a1200a21a220000100001100x010y001z0001=a11a1200a21a2200001z0001

and

TR=100x010y001z0001a11a1200a21a220000100001=a11a1200a21a2200001z0001

and this time the results are equal, therefore T and R do commute in this case.  Since the geometry is independent of the choice of coordinate system, T and R will always commute when the translation is perpendicular to the plane of the rotation.

- (10%) Sketch the Bézier curves defined by the following control polygons.  Hint: recall the rule about midpoints and tangents.

- (10%) Sketch the Bézier blending functions Jni(t) over 0  t  1 for n = 3.  Label the curves you draw to show which is which.

- (15%) Consider the operation of rotating about an arbitrary point, p = [x, y, 1]T, in the plane.  Using 3×3 homogeneous matrices, prove that rotating about the origin and then applying the translation that moves the image of p back to its original position is equivalent to translating to the origin, performing the rotation and then translating back.

Let c = cos and s = sin.  Then Rp=c−s0sc0001xy1=cx−sysx+cy1 is

the result of rotating p about the origin by , and

T=10x−(cx−sy)01y−(sx+cy)001 takes this point back into p, so that

TR=10x−(cx−sy)01y−(sx+cy)001c−s0sc0001=c−sx−(cx−sy)scy−(sx+cy)001

but the process of translating p to the origin, rotating by  and then translating back to p is given by

To−1RTo=10x01y001c−s0sc000110−x01−y001=10x01y001c−s−(cx−sy)sc−(sx+cy)001

=c−sx−(cx−sy)scy−(sx+cy)001  and these are obviously equal,

therefore the two operations are equivalent.

- (15%) Sketch top, front and right side views of the following object.

END