ELEC 200 – Fall 2006	Midterm Solutions

- a) [12 marks] Let P be the plane passing through the point [4,3,2,1]T and normal to vector [2,3,4,1]T.  Derive a matrix, M, which transforms P so that it is parallel to the yz plane and still passes through [4,3,2,1]T.  Express M as a product of elementary transformations.

Let A be the transformation that rotates the plane so it becomes parallel to the yz plane, and let T be the translation that moves the given point to the origin.  Then

M  T-1AT

where it is immediately obvious that

T=100-4010-3001-20001   and   T-1=1004010300120001

but there are four different ways to define A, any one of which completes a correct solution to this question.  Since A rotates the plane  to become to the yz plane, it also rotates the normal, N = [2,3,4,1]T to become parallel to the x axis.  There are 4 ways of doing this:

- Rotate N by  about z into the zx plane, then by  about y into the x axis, in which case

sinθ=−34+9=−313, cosθ=213, sinφ=44+9+16=429, cosφ=1329

A=Rzx(φ) Rxy(θ)=1329042900100−4290132900001 21331300−3132130000100001

- Rotate N by  about y into the xy plane, then by  about z into the x axis, in which case

sinθ=44+16=420, cosθ=220, sinφ=−34+9+16=−329, cosφ=2029

A=Rxy(φ) Rzx(θ)=202932900−32920290000100001 220042000100−420022000001

- Rotate N by  about x into the zx plane, then by  about y into the x axis, in which case

sinθ=39+16=35, cosθ=45, sinφ=9+164+9+16=529, cosφ=229

A=Rzx(φ) Rxy(θ)=229052900100−529022900001 1000045−3500354500001

- Rotate N by  about x into the xy plane, then by  about z into the x axis, in which case

sinθ=−49+16=−45, cosθ=35, sinφ=−9+164+9+16=−529, cosφ=229

A=Rxy(φ) Rzx(θ)=22952900−5292290000100001 10000354500−453500001

b) [8 marks] Draw one or two diagrams showing the derivation of M.

The following diagram shows the plane, P, its normal, N, and the translation, T, which moves it to the origin:

The following diagram goes with case 1 above:

The following diagram goes with case 2 above:

The following diagram goes with case 3 above:

The following diagram goes with case 4 above:

- a) [5 marks] Describe in words the action of the matrix, T, where

T=10ab010c001d0001

The matrix T shears lines parallel to the x axis into lines making an angle of tan-1 a with the x axis and then translates by displacement [b, c, d]T.

b) [5 marks] Express T as a product of elementary transformations.

T=100b010c001d0001 10a0010000100001

- a) [5 marks] Given a quadrilateral, ABCD, lying in the xy plane as shown below, derive the transformation matrix, T, that transforms ABCD into A'B'C'D' and express T as a product of elementary transformations.

The figure A'B'C'D' is obtained from ABCD by a rotation of  radians about the origin.  No other terms are required, so T is a trivial single-term product:

T=cos π-sin π0sin πcos π0001

b) [5 marks] Assuming the gridlines in the above figure have unit spacing, calculate the matrix, T, numerically.

Since sin  = 0 and cos = -1, we have

T=cos π-sin π0sin πcos π0001=-1000-10001

- [20 marks] Using 3D homogeneous coordinates, write a MatLab function that takes no parameters and returns a 4 × 4 matrix that transforms the the line from [1,0,0,1]T to [0,1,0,1]T into the line from the origin to [1,1,1,1]T.

There are two ways to do this problem.  Both begin by translating the starting point of the line to the origin and then rotating the line to align it with a coordinate axis.  Any axis will do, so we choose the simplest, which is the y axis.  Once the line has been brought into coincidence with the y axis, we can either scale it to length and rotate it up, or shear it up to bring its endpoint to a height of 1 above the xy plane.  Finally, we rotatate the resulting line by /4 radians to put it in its final position.  Either method is correct.

% --------------------------------------------------------------

% MakeTransform – Return a 4 x 4 homogeneous transform matrix

%       that takes the line from [1,0,0,1]T to [0,1,0,1]T into

%       the line from [0,0,0,1]T to [1,1,1,1]T using scaling and

%       rotation.

%

% Args: none

%

% Rtns: A - 4 x 4 homogeneous transform matrix

% --------------------------------------------------------------

function A = MakeTransform()

T = [ % translate starting point to the origin

1  0  0 -1

0  1  0  0

0  0  1  0

0  0  0  1

];

c = 1/sqrt(2); s = -1/sqrt(2);

R1 = [ % rotate about z by –pi/4 to align with the y axis

c -s  0  0

s  c  0  0

0  0  1  0

0  0  0  1

];

a = sqrt(3)/sqrt(2);

Sc = [ % scale the line to make length = sqrt(3)

1  0  0  0

0  a  0  0

0  0  1  0

0  0  0  1

];

c = sqrt(2)/sqrt(3); s = 1/sqrt(3);

R2 = [ % rotate about x to make z = 1 at the endpoint

1  0  0  0

0  c -s  0

0  s  c  0

0  0  0  1

];

R3 = R1; % rotate another -pi/4 to the final position

A = R3 * R2 * Sc * R1 * T; % compose the transformations

end

% --------------------------------------------------------------

% MakeTransform – Return a 4 x 4 homogeneous transform matrix

%       that takes the line from [1,0,0,1]T to [0,1,0,1]T into

%       the line from [0,0,0,1]T to [1,1,1,1]T using shear.

%

% Args: none

%

% Rtns: A - 4 x 4 homogeneous transform matrix

% --------------------------------------------------------------

function A = MakeTransform()

T = [ % translate starting point to the origin

1  0  0 -1

0  1  0  0

0  0  1  0

0  0  0  1

];

c = 1/sqrt(2); s = -1/sqrt(2);

R1 = [ % rotate about z by –pi/4 to align with the y axis

c -s  0  0

s  c  0  0

0  0  1  0

0  0  0  1

];

a = 1/sqrt(2);

Sh = [ % shear along z to make z = 1 at the endpoint

1  0  0  0

0  1  0  0

0  a  1  0

0  0  0  1

];

R3 = R1; % rotate another -pi/4 to the final position

A = R3 * Sh * R1 * T; % compose the transformations

end